Homelab 42U Rack Layout

Enterprise-Grade Home Network Infrastructure

Reference Architecture for VLAN-Segmented Homelab

Architecture Overview

This rack layout demonstrates a professional homelab setup with:

• OPNsense firewall with 7-VLAN network segmentation

• Proxmox VE cluster (2 nodes) with dedicated 10G cluster network

• NAS storage with NFS for virtualization

• UniFi network management with PoE WiFi access points

• Proper power distribution with UPS battery backup

• Clean cable management with patch panel

Equipment Legend:

| Power | Network | Compute | Storage | Firewall |
| --- | --- | --- | --- | --- |

Rack Layout (Top to Bottom)

| U Position | Equipment | Purpose |
| --- | --- | --- |
| U42-U39 | Blank Panels | Aesthetics |
| U38 | 24-Port Cat6a Patch Panel | Cable management |
| U37 | Managed PoE Switch (24-port) | Network distribution |
| U36 | Blank Panel |  |
| U35 | OPNsense Firewall Router | WAN + 7 VLANs |
| U34 | Blank Panel |  |
| U33 | Proxmox Cluster Nodes (×2) | Virtualization |
| U32-U30 | Blank Panels |  |
| U29 | NAS (4-Bay) | NFS storage |
| U28-U20 | Blank Panels |  |
| U19 | Horizontal PDU | Power distribution |
| U18-U4 | Blank Panels | Future expansion |
| U3-U2 | Rackmount UPS (2U) | Battery backup |
| U1 | Blank Panel |  |

Network Topology

WAN Connection:

• ISP Gateway (bridge mode) → OPNsense WAN port

• External cable run through building exterior

LAN Topology:

• OPNsense LAN → Patch Panel → Switch Port 1 (VLAN trunk)

• All equipment connected through patch panel for clean management

VLAN Segmentation (7 VLANs):

• VLAN 1: Management (infrastructure devices)

• VLAN 10: Trusted (personal devices)

• VLAN 20: Servers (VMs and containers)

• VLAN 30: Media (Plex, Home Assistant)

• VLAN 40: IoT (smart home devices, restricted)

• VLAN 50: Lab (testing and development)

• VLAN 99: Guest (internet-only, fully isolated)

Cluster Network:

• Proxmox nodes connected via direct 10G SFP+ link (DAC cable)

• Dedicated network for VM migration and Corosync

• Bypasses switch for maximum performance

WiFi Access Points:

• Primary AP: Ceiling-mounted (WiFi 7, 2.5G PoE+ uplink)

• Secondary AP: Basement coverage

• Multiple SSIDs mapped to VLANs (Trusted, IoT, Guest)

Cable Plan

| Qty | Cable Type | Length | Purpose |
| --- | --- | --- | --- |
| 1 | Cat6 | 25-30ft | WAN: ISP gateway to firewall (exterior run) |
| 6 | Cat6a patch | 1ft | Rack equipment to patch panel |
| 6 | Cat6a patch | 1ft | Patch panel to switch |
| 2 | Cat6a | 25-50ft | WiFi AP uplinks |
| 1 | SFP+ DAC | 1ft | Proxmox cluster direct link (10G) |
| 1 | Power | 10ft | PDU to UPS connection |
| 6 | Power (C13) | varies | Equipment to PDU |
| 1 | Power cord | 6ft | UPS to wall outlet |

Design Best Practices

Weight Distribution:

• Heaviest equipment (UPS) positioned at bottom for stability

• Prevents rack tipping and reduces stress on mounting points

Cable Management:

• Patch panel at top of active equipment section

• Short 1ft patches inside rack for clean appearance

• All cables routed through rear vertical cable management

• Velcro straps (not zip ties) for flexibility

Airflow & Cooling:

• Blank panels prevent hot air recirculation

• Equipment spacing allows proper ventilation

• Cool air intake from front, hot air exhaust to rear

Power Management:

• All critical equipment on UPS battery backup

• Mid-rack PDU for convenient power distribution

• Clean power cable routing to rear of rack

Scalability:

• 15U reserved for future expansion (U18-U4)

• Switch has ports available for additional devices

• VLAN architecture supports easy network segmentation

Technology Stack

Firewall/Router:

• OPNsense (open-source firewall platform)

• 7-VLAN network segmentation

• WireGuard VPN for secure remote access

Virtualization:

• Proxmox VE cluster (2 nodes)

• High availability configuration

• 10G cluster network for VM migration

Storage:

• NAS with NFS for Proxmox VM storage

• RAID configuration for data protection

• Future 10G upgrade path available

Network Management:

• Ubiquiti UniFi ecosystem

• Centralized controller management

• PoE-powered WiFi access points

---

This is a reference architecture for educational purposes.

Adapt network addresses, VLANs, and equipment to your specific requirements.